| MARUTI SUZUKI INDIA LIMITED |  | WORK INSTRUCTION SHEET |  |
| --- | --- | --- | --- |
| Document No. | WIS-ENG-MNT-SW-0421 | Revision | B |
| Date | 18-Sep-2023 | Prepared By | R. Sharma / QE-DE-DX3 |

1. PART IDENTIFICATION

| Part Name | Engine Mount Bracket (Front LH) | Drawing No. | MSIL-DE-SW-0421-C |
| --- | --- | --- | --- |
| Part Number | SW-ENG-MNT-001-B | Drawing Rev. | B |
| Vehicle Model | Maruti Swift 1.2L K-Series | Plant | Manesar |

2. SCOPE

This Work Instruction covers the assembly, torque application, and quality inspection of the Engine Mount Bracket (Front LH) — ENG-MNT-FLH-K12B — on the Maruti Swift 1.2L K-Series engine line at Manesar Plant.

3. TOOLS & EQUIPMENT REQUIRED

| Item | Description | Specification / Code |
| --- | --- | --- |
| 1 | Torque Wrench | TL-BRK-001-OLD ¦ Range: 10–100 Nm |
| 2 | M10 Hex Socket (T25) | STD-HEX-M10-T25 |
| 3 | Surface Plate | Grade B, 600×400 mm |
| 4 | Bore Gauge | Range: 50–75 mm, Res. 0.001 mm |
| 5 | Surface Roughness Tester | Mitutoyo SJ-210, Ra range 0.05–16 µm |
| 6 | Dial Indicator | Resolution 0.01 mm, Travel 10 mm |

4. MATERIAL SPECIFICATION

| Material Grade | Thickness | Heat Treatment | Surface Treatment |
| --- | --- | --- | --- |
| IS 2062 Gr. B | t = 4.0 mm ±0.15 | NIL | Phosphate + Paint |

5. ASSEMBLY SEQUENCE

| Step | Operation | Acceptance Criteria |
| --- | --- | --- |
| 1 | Inspect bracket body SW-ENG-MNT-001-B for burrs, cracks, and surface defects. Deburr sharp edges per note 7. | No visible defects. All edges deburred. |
| 2 | Verify bore ID ø64.00 mm using bore gauge. Acceptance: ø64.00 ±0.05 mm. | Bore within ±0.05 mm of nominal |
| 3 | Position bracket on assembly jig. Align to datum A (top face), datum B (left edge), datum C (centre bore axis). | Datums seated flush, no tilt. |
| 4 | Install 4× M10 hex bolts Gr. 8.8 (STD-M10-HEX-8.8) with flat washers (STD-M10-WSH-FLT). Apply thread-locking compound if specified. | All fasteners hand-tight before torque. |
| 5 | Apply torque to M10 bolts in cross-pattern sequence: Stage 1 = 25 Nm, Stage 2 = 45 Nm (final). | Torque: 45 Nm ±5% per bolt |
| 6 | Verify surface finish on machined faces. Acceptance: Ra ≤ 3.2 µm. | Ra ≤ 3.2 µm (machined faces) |
| 7 | Check flatness of top face (Datum A) using dial indicator on surface plate. Acceptance: ≤ 0.10 mm. | Flatness ≤ 0.10 mm |
| 8 | Verify true position of centre bore ⌀0.08 M relative to datums A, B, C using CMM or optical comparator. | True position ≤ ⌀0.08 mm |
| 9 | Verify parallelism of mating face to Datum A. Acceptance: ≤ 0.15 mm. | Parallelism ≤ 0.15 mm |
| 10 | Record all measurement results in Quality Inspection Report (QIR-ENG-MNT-SW). Sign off and release bracket to next station. | QIR completed and signed. |

6. TORQUE SPECIFICATIONS

| Fastener | Size | Grade | Torque (Nm) | Sequence |
| --- | --- | --- | --- | --- |
| M10 Hex Bolt (Bracket-to-Block) | M10 | 8.8 | 45 Nm ±5% | Cross-pattern, 2-stage |
| Rubber Isolator Retainer | M8 | 8.8 | 22 Nm ±5% | Sequential |

⚠  NOTE: Apply torque in two stages. Stage 1: 25 Nm (snug). Stage 2: 45 Nm (final). Do NOT exceed specified value. Re-torque not required after first assembly.

7. DIMENSIONAL ACCEPTANCE CRITERIA

| Parameter | Nominal | Tolerance (WIS) | Instrument |
| --- | --- | --- | --- |
| Bore Diameter | ø64.00 mm | ±0.05 mm | Bore Gauge |
| Bore Depth | 38.5 mm | ±0.1 mm | Depth Micrometer |
| Overall Width | 300 mm | ±0.2 mm | Vernier Caliper |
| Overall Height | 180 mm | ±0.2 mm | Vernier Caliper |
| Gap / Slot Width | 260 mm | ±0.05 mm | Vernier Caliper |
| Plate Thickness | 4.0 mm | ±0.15 mm | Micrometer |
| Flatness (Top Face) | — | ≤ 0.10 mm | Dial Indicator on Surface Plate |
| Parallelism (Mating Face) | — | ≤ 0.15 mm | Dial Indicator |
| True Position (Bore) | — | ⌀0.08 mm | CMM |
| Surface Finish (Machined) | — | Ra ≤ 3.2 µm | Surface Roughness Tester |

8. REVISION HISTORY

| Rev. | Date | Description | Approved By |
| --- | --- | --- | --- |
| A | 03-Jun-2023 | Initial release | S. Kumar |
| B | 18-Sep-2023 | M6 pilot holes added; tooling updated to TL-BRK-001-OLD | R. Sharma |

This document is the property of Maruti Suzuki India Limited. Unauthorized reproduction or distribution is prohibited.  |  MSIL_DE_DX3  |  CONFIDENTIAL  |  Rev. B  |  WIS-ENG-MNT-SW-0421